Proposed Changes to 61970 ACLineSegment modeling for mutual coupling

From the Device Datasheet Breakout Group

30 January, 2022

Changes to UML classes, attributes, and associations

61970 changes

Add class LineSegmentCoupling, a child of IdentifiedObject

with attributes

.coupledLineNumber

.reverseFlow

.xOffset

Add class CoupledLineSegmentGroup, a child of IdentifiedObject

with no attributes

Add association LineSegmentCoupling.ACLineSegment

Add association LineSegmentCoupling.CoupledLineSegmentGroup

Changes to UML descriptions

Associations

## LineSegmentCoupling to ACLineSegment association

SOURCE LineSegmentCoupling

Role: LineSegmentCoupling

The description of this line's coupling with other lines in a coupled group.

Multiplicity: [0..1]

TARGET ACLineSegment

Role: ACLineSegment

The line whose relationship in a coupled group is described.

Multiplicity: [1]

## LineSegmentCoupling to CoupledLineSegmentGroup

SOURCE LineSegmentCoupling

Role: LineSegmentCoupling

A coupled line in this coupled group of lines.

Multiplicity: [1..*]

TARGET CoupledLineSegmentGroup

Role: CoupledLineSegmentGroup

The coupled group of lines with which this coupled line has relationships.

Multiplicity: [1]

Classes and Attributes

## LineSegmentCoupling

Class in package 'Wires'

Describes the relationship of a line in a coupled group to the reference line in the group. (Reference line has a coupledLineNumber = 1.)

ATTRIBUTES

coupledLineNumber : Integer Public Multiplicity: [0..1] Sequence number of coupled line. Value of 1 indicates that line is the reference line. Valid value range is from 1 to number of coupled lines in the group.

reverseFlow : Boolean Public Multiplicity: [0..1] Indication of whether the reference end of a line in a coupled group is the same as the reference end of the reference line. Set reverseFlow to True for any lines that don’t have their Terminal with sequenceNumber = 1 at the same end as the reference line. Set reverseFlow to False for the reference line and for any lines that have their Terminal with sequenceNumber = 1 at the same end as the reference line. The sign of the coupled voltage drop of an ACLineSegment reverses when two coupled lines have their Terminals with sequenceNumber = 1 at opposite ends.

xOffset : Displacement Public Multiplicity: [0..1] The horizontal offset between the horizontal reference point of a coupled line and the horizontal reference point of the reference line in the group. The horizontal reference point for a line is established by the line's WireAssembly. It is the zero x position on which the xCoord values of WirePositions in the WireAssembly are based.

## CoupledLineSegmentGroup

Class in package 'Wires'

Aggregates a set of line segments that are on the same tower, or in the same right-of-way, close enough that mutual coupling impedances between the lines need to be included in network analysis.